Feedback in systems [handling negative/positive feedback]

Mark McIlroy, March 2010

Crashes and evolutions of systems to crashes

Negative feedback items (the next change is back from the distance away) - oscillation

Positive feedback items (the next change is added to the distance away)– spiraling out of path to a crash

Positive feedback

- Calculate the incremental move each time period, discard it don’t add it to the previous value unless a flag is set

Standard positive feedback			adjusted system

crash

- Place a limit or maximum on an individual element, to prevent all value flowing to a single element and all other elements going to zero.

For this solution to be effective, the pressure must be pushed back to its original source, not taken and directed elsewhere.

Negative feedback

Moving from level A to level B

Oscillation – object has momentum as it passes through level B, causing it to continue moving through the level to beyond it then returning back to form oscillation.

Parameters (space shuttle docking analogy)

a. Initial velocity (at right angles to the surface)

b Switch-on time positive thrust (increasing speed)

c Drift time (constant speed)

d Switch-on time negative thrust (decreasing speed)

e Speed at impact (set to zero for adjustment of aircraft flight to a new level, a small value for docking physical items)

Simulate values for all parameters to achieve movement from level A to level B effectively

a = f / m

x = x + vt

v = v + a

Drift

Adjusting a system to maintain a constant value for a parameter.

A problem with continous adjustment is that oscilation with increasing amplitude may occur if there is a delay in the feedback loop.

Level of flight/system parameter = A

- plan for very slow descent from 40k to 39k then slow ascent from 39k to 40k, change of direction is triggered at 39k/40k, assumes accuracy of height of +/- 0.2k for example to prevent random frequent changes.

The path is then a triangular pattern with self-correcting for drift

- stay at a level height, changing from ascent/decent/level at points in time, e.g. every 3 minutes, not continuously

Drift. 2

Reference a value to the original reference (inflated if necessary), not the previous value.

e.g.

value/error referenced to previous value			value/error referenced to starting value

x +- 0.5								x +- 0.5

x +- 1.0								x +- 0.5

x +- 1.5								x +- 0.5

x +- 2.0								x +- 0.5

x +- 2.5								x +- 0.5

.

.

;

e.g. social security payments from 1971 to present, adjustment for inflation

Explosions / spread

The risk of a small change converting an entire system from one for to another.

Inks bleeding into another color tank, infection spreading through the entire barrel in barrels of fruit, explosions (conversion of items from one form to another).

Split the volume into separate cells, discard or replace damaged cells.